Assignment 3 :

1.Create a comparison table listing the main differences between DDR3, DDR4, and DDR5 RAM, including speed, voltage, and typical use cases.(DOUBLE DATA RATE)

| Feature | DDR3 | DDR4 | DDR5 |
| --- | --- | --- | --- |
| Release Year | 2007 | 2014 | 2020 |
| Speed (JEDEC Standard) | 800–2133 MT/s | 1600–3200 MT/s | 4800–8800 MT/s |
| Voltage | 1.5 V | 1.2 V | 1.1 V |
| Bandwidth | Up to ~17 GB/s | Up to ~25.6 GB/s | Starts at ~38.4 GB/s and higher |
| Maximum Module Capacity | Up to 16 GB | Up to 64 GB | 128 GB and beyond |
| Power Efficiency | Lowest | Better than DDR3 | Best among the three |
| Latency | Lower absolute latency | Moderate | Higher latency numbers but much faster overall due to increased speed |
| Error Correction | No on-die ECC | No on-die ECC | On-die ECC (improves reliability internally) |
| Compatibility | DDR3-only motherboards | DDR4-only motherboards | DDR5-only motherboards |
| Physical Notch | Different position | Different from DDR3 | Different from DDR4 |
| Typical Use Cases | Older desktops and laptops | Mainstream PCs, gaming, office work | High-end gaming, content creation, AI workloads, future PCs |

3.Open your laptop or desktop (with power disconnected), locate the RAM slots, and take a clear photo of the installed RAM module. Reinstall the RAM module and verify that your system boots up successfully.<br><br><em><strong>Hint:</strong> Refer to your device manual or a trusted YouTube guide if unsure about RAM removal and installation steps.</em>

3.List three key differences between HDD, SSD, and NVMe storage types, and for each, name one app or use case (from apps you use daily) that would benefit most from that storage type.

| Feature | HDD |  |  | SSD (SATA) | NVMe SSD |
| --- | --- | --- | --- | --- | --- |
| Speed | Slow (80–160 MB/s) |  |  | Fast (500–550 MB/s) | Very Fast (2,000–7,000+ MB/s) |
| Technology | Spinning magnetic disks |  |  | Flash memory (no moving parts) | Flash memory using PCIe Interface |
| Best Use Case | Large, low-cost storage |  |  | Everyday Windows use and applications | High-performance tasks like gaming, video editing, and large file transfers |

| Storage Type | Daily App or Use Case | Why It Benefits |
| --- | --- | --- |
| HDD | Movies, Photos, Music collection | Large storage capacity at a low cost. |
| SSD (SATA) | Google Chrome / Microsoft Office | Faster startup and quicker loading of applications. |
| NVMe SSD | Adobe Premiere Pro (or modern games like BGMI emulators/AAA PC games) | Extremely fast loading, file transfers, and project processing. |

4. Remove the storage drive (HDD or SSD) from your system, take a photo of the connector type (SATA, M.2, or NVMe), and reinstall the drive. Confirm that your device recognizes the storage after reassembly.<br><br><em><strong>Constraint:</strong> Only perform this task if you have access to a device you are allowed to open. If not, find and share a YouTube video link showing the process for your laptop/PC model.</em>